Work Instruction for HPE Firmware Upgrade

Viatris

Work Instruction for HPE Firmware Upgrade

# Contents

Contents	2

1. Introduction	3

1.1. Purpose	3

1.2. Scope of this Report	3

1.3. Audience	3

2.0 Approach and Methodology	3

2.1 Approach	3

3. Technical Details	4

4.	Roles and Responsibilities	14

5. Escalation Matrix	14

6. Related Documentation	15

6.1. Additional Documentation	15

7. Version Control	15

# 1. Introduction

## 1.1. Purpose

This document will provide the procedure to be followed for HPE firmware upgrade.

## 1.2. Scope of this Report

What this document covers:

The purpose of this guide is to provide details on how HPE firmware upgrade.

Usage Expectations:

This document covers end-to-end support guidelines for performing HPE firmware upgrade.

## 1.3. Audience

GIDC Unix Group

# 2.0 Approach and Methodology

## . Approach

    - Backup the operating system (Boot disk).

    - Download the SOS report and AHS logs from HPE server.

    - Subscribe the OS with RedHat portal

# subscription-manager refresh

# subscription-manager list

# yum clean all

# yum repolist

2.2. Methodology

    - Download the required ISO from the HPE portal and mount it on the server console.

    - Reboot the server and select the necessary fields in the console and ISO.

    - Select the type of firmware upgrade and reboot the server after completing the deployment.

    - Login to the RHEL, update and reboot the O.S.

# 3. Technical Details

3.1. Step 1

3.1.1 Download the SSP ISO from the HPE portal

https://support.hpe.com/connect/s/softwaredetails?language=en_US&softwareId=MTX_50b300127e24428582ca72564a&_sm_au_=iVVWn4SqtFS8V8nqHGJk8Ks61Hf31

3.1.2 Mount the SSP ISO on the server console

3.1.3 Reboot the server and select F11 (Boot Menu) in the console

3.2. Step 2

3.2.1 Select One Time Boot Menu

3.2.2 Select the CD-ROM to boot from ISO

3.2.3 There are two ways to update the firmware

- Automatic Firmware upgrade à System will update all the required packages required

- Interactive Firmware upgrade à Need to select the required packages to install

3.3. Step 3

3.3.1. Reboot the server after completing the Deployment

3.3.2. Login to the RHEL and update the operating system

3.3.3. Reboot the operating system after OS update.

# Roles and Responsibilities

| Sl. No. | Roles | Responsibilities | Team Member Name |
| --- | --- | --- | --- |
| 1. | Maintenance Window | Respective Team / Unix Team | GIDC.unix_Run GIDC.unix_Run@viatris.com |
| 2. | Change Request | Respective Team / Unix Team |  |
| 3. | Change Implementation | Respective Team / Unix Team |  |
| 4. | Hyper Care | Respective Team / Unix Team |  |

# 5. Escalation Matrix

| Person Responsible | Role | Triggers When |
| --- | --- | --- |
| Arun R Arun.R@viatris.com +91 9710677240 | SME |  |
| Ashok Kumar R K AshokKumar.RK@viatris.com +91 9967735890 | Manager |  |
| Mohandoss Murugesan Mohandoss.Murugesan@viatris.com +91 6366966170 | Senior Manager |  |

# 6. Related Documentation

## 6.1. Additional Documentation

# 7. Version Control

| Date | Version | Summary of Changes | Author | Approved by |
| --- | --- | --- | --- | --- |
| July 28 2023 | V1.0 | Updated Organization Detail | Ashok Kumar R K | Mohandoss Murugesan |
| Dec 28 2024 | V1.1 | Validated the Document | Benesh Selva Nesan P | Arun R |