Work Instruction for Adding ASM Disks in Virtual Server

Viatris

Work Instruction for Adding ASM Disks in Virtual Server

# Contents

Contents	2

1. Introduction	4

1.1. Purpose	4

1.2. Scope of this Report	4

1.3. Audience	4

2. Approach and Methodology	4

2.1.	Approach	4

2.1.1. Capacity growth check	4

2.1.2. Create a Change Request	4

2.1.3. Assign Virtual disk to the server from Vcenter	4

2.1.4. Scan for Newly Added Disks	4

2.1.5. Backup Existing ASM Configuration Rules	4

2.1.6. Partition the New Disks	4

2.1.7. Update ASM Rules	5

2.1.8. Reload ASM Rules	5

2.1.9. Verify Disk Visibility in ASM	5

2.1.10. Confirm Change Approval	5

2.2.	Methodology	5

2.2.1. Planning and Preparation	5

2.2.2. Pre-Implementation Validation	5

2.2.3. Disk Configuration	5

2.2.4. Post-Implementation Validation	6

2.2.5. Documentation and Compliance	6

2.2.6. Rollback Plan	6

3.	Technical Details	6

3.1.	Add the required disk from Vcenter	6

3.2.	Scan for the newly added disk	6

3.3.	Backup Existing ASM Configuration Rules	7

3.4.	Partition the New Disks	7

3.5.	Find the UUID (Universally Unique Identifier) for the block device	7

3.6.	Update ASM Rules	7

3.7.	Reload ASM Rules	8

3.8.	Verify Disk Visibility in ASM	8

4.	Roles and Responsibilities	8

5.	Escalation Matrix	8

6. Revision History	9

# 1. Introduction

## 1.1. Purpose

This document will provide the procedure to be followed for adding ASM disks to the Virtual server based on the requirement.

## 1.2. Scope of this Report

The purpose of this guide is to provide details on adding ASM disks to the  virtual server.

## 1.3. Audience

GIDC UNIX Group

# 2. Approach and Methodology

## Approach

## 2.1.1. Capacity growth check

- Review server capacity growth. If the increase is ≤10%, proceed without approval.

If the increase exceeds 10%, obtain intake approval.

#### 2.1.2. Create a Change Request

- Create a Change Request (CR) in ServiceNow for adding ASM disks.

#### 2.1.3. Assign Virtual disk to the server from Vcenter

- Add a new virtual disk with the required size to the server from vCenter.

#### 2.1.4. Scan for Newly Added Disks

- Scan the server for the newly added disks and verify their sizes.

#### 2.1.5. Backup Existing ASM Configuration Rules

- Backup the current ASM configuration rules located in   /etc/udev/rules.d.

#### 2.1.6. Partition the New Disks

- Create partitions for the newly added disks using tools such as fdisk or  parted.

#### 2.1.7. Update ASM Rules

- Add entries for the newly added disks in the /etc/udev/rules.d/<ASM Rules> file.

#### 2.1.8. Reload ASM Rules

- Reload the ASM rules using the udevadm control --reload-rules and udevadm trigger commands.

#### 2.1.9. Verify Disk Visibility in ASM

- Check if the newly added disks are visible in the ASM disk group.

#### 2.1.10. Confirm Change Approval

- Ensure that the change management process is completed and the  change request is approved for implementation.

## Methodology

#### 2.2.1. Planning and Preparation

- Change Request (CR): Create and document a CR in ServiceNow with all relevant details and obtain approvals.

#### 2.2.2. Pre-Implementation Validation

- Disk Availability:

Scan for the newly allocated disks and verify their sizes using tools like lsblk or fdisk.

- Backup Configuration:

Back up the existing ASM configuration rules in /etc/udev/rules.d to allow easy rollback if needed.

#### 2.2.3. Disk Configuration

- Partition the Disks:

Use partitioning tools (fdisk, parted) to create the necessary partitions for ASM.

- Update ASM Rules:

Modify the /etc/udev/rules.d/<ASM Rules> file to include new disks, ensuring correct ownership, permissions, and naming.

- Apply Changes:

Reload the udev rules using udevadm control --reload-rules and udevadm trigger.

#### 2.2.4. Post-Implementation Validation

- Log Review:

Check ASM and system logs for any warnings or errors related to the new disks.

#### 2.2.5. Documentation and Compliance

1. Document the implementation process with detailed steps,     commands, and screenshots where applicable.

2. Ensure compliance with organizational change management and auditing requirements.

#### 2.2.6. Rollback Plan

Maintain a clear rollback plan, leveraging the backups of the ASM configuration and removing the newly added disks if required.

# Technical Details

## Add the required disk from Vcenter

Log in to the vCenter with SADM account and navigate to the desired server. Then follow these steps:

- Click on Actions > Edit Settings.

- Under ADD NEW DEVICE, select Hard Disk.

- In the Hard Disk section, a new row for the hard disk will appear.

- Specify the desired size for the new hard disk in the appropriate field.

- Click OK to save the changes.

## Scan for the newly added disk

Login to the server using MID and password then switch to root user using sudo su - command and use the command lsblk to check the newly added disk from Vcenter end is visible in the server

# Use lsblk to list the block devices and their sizes

# Use fdisk -l <Block_Device> to confirm the disk details

## Backup Existing ASM Configuration Rules

Take backup of the configuration file /etc/udev/rules.d/99-oracle asmdevices.rules

#cp /etc/udev/rules.d/99-oracle-asmdevices.rules /etc/udev/rules.d/99-oracle-  asmdevices.rules_bkp_`date +'%d%m%Y'`

## Partition the New Disks

- Once the new disk has been added create the partition

# fdisk [block device]

· To Create a new partition Press n and press Enter

· Choose primary partition by type p and press Enter.

· Choose Partition number by type 1 and press Enter

· Choose Partition cylinders start value as default and press Enter

· Choose Partition cylinders end value as default and press Enter

· Change the partition type by type t and press Enter

· Choose partition number by type 1 and press Enter

· View the partition table by type 'p' and press Enter

· Save the Partition table by type 'w' and press Enter

## Find the UUID (Universally Unique Identifier) for the block device

# /usr/lib/udev/scsi_id -u -g -d [block device]

## Update ASM Rules

Open the oracle-asmdevices rules file and add the below lines with new disk UUID as obtained in the previous step

# vi /etc/udev/rules.d/99-oracle-asmdevices.rules

ACTION=="add|change", ENV{DM_UUID}=="part1-mpath-<UUID(LUN_ID)>", SYMLINK+="<ASM DISK Name>", GROUP="asmadmin", OWNER="grid", MODE="0660"

Note: Collect the UID ids and update above configuration as well us ASM Disk Name from the Change Request Work Notes

## Reload ASM Rules

- Reload the udev rule and update in the kernel

# udevadm control --reload-rules

# udevadm trigger --type=devices --action=change

## Verify Disk Visibility in ASM

- Validate the newly created ASM disks using the following command:

# ls – l /dev/asm*

# Roles and Responsibilities

| Sl. No. | Roles | Responsibilities | Team Member Name |
| --- | --- | --- | --- |
| 1. | Change Request | Respective Team / Unix Team | GIDC.unix_Run GIDC.unix_Run@viatris.com |
| 2. | Change Implementation | Respective Team / Unix Team |  |

# Escalation Matrix

| Person Responsible | Role | Triggers When |
| --- | --- | --- |
| Arun R Arun.R@viatris.com +91 9710677240 | Deputy Manager |  |
| Ashok Kumar R K AshokKumar.RK@viatris.com +91 9967735890 | Senior Manager |  |
| Zahir Hussain M ZahirHussain.M@viatris.com +91 9840689462 | Senior Manager |  |

# 6. Revision History

| Date | Version | Summary of Changes | Author | Approved by |
| --- | --- | --- | --- | --- |
| April 28 2025 | V1.0 | Created the Document | Benesh Selva Nesan P | Arun R |
| April 28 2026 | V1.1 | Validated the Document | Benesh Selva Nesan P | Arun R |